## title: ESP32 OTA (ESP_32_OTA) Mechanism version: '1.0' commit: '51e257c695009957ec1183a0f769b73908d4013b' branch: 'DT' date: '2025-11-15'

# ESP32 OTA (ESP_32_OTA) Mechanism â€” Comprehensive Documentation

Version: 1.0

Repository: d:\LabExpert\LabExpert_Sensor_Communications

Build Metadata:

- Commit: <to-be-populated>

- Branch: <to-be-populated>

- Build date: <to-be-populated>

## Overview

- Purpose: Provide a robust OTA process for ESP32-based devices, featuring a dedicated OTA bootloader (ota_0) and application firmware (ota_1).

- Components: ESP32 OTA bootloader (ESP_32_OTA), sensor firmwares (TOF_*, UltraSonic_*, OSI_*), backend service, browser client, WiFi network, MQTT broker, UDP discovery.

- High-level modes:

  - Bootloader mode (ota_0): Accepts firmware via HTTP upload and a backend push API; performs partition erase and streaming writes, then restarts.

  - Application mode (ota_1): Runs sensor/UI firmware; can trigger rollback to bootloader upon sensor/EEPROM failure or manual input.

## Workflow Analysis

### Initialization

- Partition status: Logs current partition at boot and treats ota_0 as OTA bootloader mode (ESP_32_OTA/src/main.cpp:391-399).

- Sensor detection: Reads EEPROM over I2C (Wire) with retries; sets sensorType or marks UNKNOWN and handles fallback (ESP_32_OTA/src/main.cpp:94-156).

- WiFi setup: STA mode, DHCP, connect to ssid/password, then start WebServer and UDP discovery (ESP_32_OTA/src/main.cpp:454-483).

- Inactive partition erase: At startup, erases the non-running OTA partition to ensure a clean target (ESP_32_OTA/src/main.cpp:66-92, called at 450-452).

- Route setup: Registers HTTP routes including OTA web upload and backend push endpoints (ESP_32_OTA/src/main.cpp:196-339).

### Firmware Download Sequence

- Web upload (multipart form):

  - POST /update writes file chunks: Update.begin(UPDATE_SIZE_UNKNOWN) â†’ Update.write(upload.buf) â†’ Update.end(true), then restart (ESP_32_OTA/src/main.cpp:211-242).

- Backend push API (JSON hex streaming):

  - POST /ota/begin { size } initializes Update.begin(size) and sets otaInProgress (ESP_32_OTA/src/main.cpp:265-287).

  - POST /ota/write { offset, size, data(hex) } decodes hex â†’ bytes and calls Update.write(bytes) with strict size checks (ESP_32_OTA/src/main.cpp:288-321).

  - POST /ota/end finalizes with Update.end(true) and restarts (ESP_32_OTA/src/main.cpp:322-339).

- Sensor firmwares (ESPAsyncWebServer):

  - POST /update handler streams Update.write(data) and finalizes Update.end(true) then restart (TOF_Firmware_bin_Generator/src/config_handler.cpp:212-241).

### Verification Steps

- Write verification:

  - Checks Update.begin and compares bytes written to expected size; errors via Update.printError(Serial).

- Completion status:

  - Uses Update.end(true) and Update.hasError() to determine OK vs FAIL (ESP_32_OTA/src/main.cpp:213, 236-242).

- Input validation for push API:

  - JSON parse checks with error codes; hex decoding validation; size mismatch guards (ESP_32_OTA/src/main.cpp:270-307).

- Cryptographic verification:

  - Not implemented; no SHA256/CRC signature checks; relies on Update success.

### Update Procedure

- Target partition selection:

  - Bootloader reports running and next update partitions; streams to esp_ota_get_next_update_partition(NULL) (ESP_32_OTA/src/main.cpp:217-221, 227-229).

- Partition erase:

  - Erases inactive partition at boot for a clean slate (ESP_32_OTA/src/main.cpp:66-92).

- Restart:

  - On success, calls ESP.restart() after brief delay (ESP_32_OTA/src/main.cpp:333-334, 221).

### Rollback Mechanism

- Manual rollback:

  - Application firmwares can disconnect MQTT/WiFi and set boot partition to the opposite OTA slot via esp_ota_set_boot_partition, then restart (OSI_Firmware_bin_Generator/src/main.cpp:205-271).

- Sensor-triggered rollback:

  - On EEPROM/sensor failure, erase inactive partition and reboot to bootloader (ESP_32_OTA/src/main.cpp:510-519).

- Automatic runtime rollback framework:

  - Not used (e.g., esp_ota_mark_app_valid_cancel_rollback absent). Rollback is explicit/manual.

## Visual Documentation

### Architecture Diagram

- See architecture.drawio and architecture.png (to be generated). Components and connections:

  - ESP32 OTA Bootloader (ota_0) â†” Browser (HTTP form upload)

  - ESP32 OTA Bootloader â†” Backend (HTTP push API)

  - ESP32 OTA Bootloader â†” UDP discovery (ports 8888/8889)

  - Application firmware (ota_1) â†” MQTT broker (status/control)

  - WiFi AP/Router between all networked components

### Sequence Diagrams

- sequence_web_upload.mmd: Browser-based upload to /update with streaming and restart.

- sequence_backend_push.mmd: Backend push flow with begin/write/end and error cases.

### Flowchart

- flowchart.mmd: End-to-end OTA process with decision points (WiFi, sensor presence, begin/write/end success, restart).

### State Diagram

- state_diagram.mmd: Bootloader â†” Main firmware â†” OTA In Progress â†” Error/Recovery states.

## Technical Specifications

### Communication Protocols

- HTTP:

  - Bootloader: WebServer on port 80. Endpoints: /, /update, /info, /ping, /id, /ota/begin, /ota/write, /ota/end (ESP_32_OTA/src/main.cpp:196-339).

  - Sensor firmwares: ESPAsyncWebServer with POST /update (TOF_Firmware_bin_Generator/src/config_handler.cpp:212-241).

- UDP:

  - Discovery port 8888, response 8889 (ESP_32_OTA/src/main.cpp:59-65, 341-389, OSI_Firmware_bin_Generator/src/main.cpp:24-31).

- MQTT:

  - Used in application firmwares for telemetry/control; not used for OTA data transfer (OSI_Firmware_bin_Generator/src/main.cpp:88-91).

### Security Measures

- Transport:

  - OTA occurs over HTTP without TLS; no esp_https_ota/WiFiClientSecure in OTA path.

- Access control:

  - OTA endpoints have no authentication; accessible on local network.

- Integrity:

  - Relies on Update success and internal write checks; no cryptographic signature or checksum verification.

- Recommendations:

  - Use HTTPS with certificate pinning for OTA push.

  - Add authentication (HMAC tokens or mutual TLS).

  - Implement firmware SHA256 verification and optional signing (ECDSA/RSA).

  - Rate-limit and CSRF-protect web upload.

### Memory Requirements and Constraints

- Partition layout (typical 4MB flash, per ESPAsyncWebServer reference table):

  - ota_0 app: ~1856 KB; ota_1 app: ~1856 KB; NVS/otadata/spiffs/coredump reserved (TOF_Firmware_bin_Generator/.pio/libdeps/esp32dev/ESPAsyncWebServer/partitions-4MB.csv:1-7).

- OTA write strategy:

  - Streaming writes consume minimal RAM; buffers are per-chunk from HTTP upload or push.

- Constraints:

  - Firmware binary must fit available app partition size.

  - Erase of inactive partition ensures maximal contiguous space.

### Performance Characteristics

- LED intervals: WiFi/Sensor LEDs toggle every 3s in bootloader (ESP_32_OTA/src/main.cpp:47-51, 159-175, 178-194).

- UDP discovery check: Every 1s in bootloader; every 5s in OSI firmware (ESP_32_OTA/src/main.cpp:63-65, 341-389; OSI_Firmware_bin_Generator/src/main.cpp:29-31, 128-183).

- Sensor check interval: 2s (ESP_32_OTA/src/main.cpp:40-42, 501-521).

- WiFi reconnection: On loss, attempts reconnect with 5s delay (ESP_32_OTA/src/main.cpp:493-499).

- OTA throughput: Bound by HTTP chunk size and WiFi RSSI; typical ESP32 SPI flash write speeds support multi-hundred KB/s in good conditions.

## API Reference Summary

- Full details in api_reference.md. Key endpoints:

  - Web upload: POST /update (multipart form-data update).

  - Backend push: POST /ota/begin, POST /ota/write, POST /ota/end.

  - Informational: GET /, GET /info, GET /ping, GET /id.

## Troubleshooting

- WiFi loss:

  - Device logs reconnection attempts; ensure SSID/password and AP stability (ESP_32_OTA/src/main.cpp:493-499).

- EEPROM/sensor not detected:

  - Bootloader may restart or remain in OTA mode; reconnect sensor and retry; check I2C wiring (ESP_32_OTA/src/main.cpp:404-425, 510-519).

- OTA write failures:

  - Inspect serial logs for Update.printError; verify chunk sizes and JSON/hex validity for push API (ESP_32_OTA/src/main.cpp:270-307, 313-318).

- Partition operations fail:

  - If esp_ota_set_boot_partition or esp_partition_erase_range fails, device falls back to restart; check partition table compatibility and flash health (ESP_32_OTA/src/main.cpp:77-86; OSI_Firmware_bin_Generator/src/main.cpp:254-271).

## Version Control Information

- firmware_version reported by bootloader in UDP discovery: OTA_BOOTLOADER (ESP_32_OTA/src/main.cpp:371).

- OSI firmware UDP discovery reports version 1.0 (OSI_Firmware_bin_Generator/src/main.cpp:158-159).

- Include commit, branch, and build date in PDF header during generation.

## Appendices

- File references used in this document follow file_path:line_number format for traceability.